Methodology

- Average battery recipe for EV battery and vehicle components

- Respective HS codes to determine imports and exports as well as the supply chain path followed

- Outlining country-specific % of that data (e.g. In 2023 75% of lithium imports in the US came from China)

- Identifying major companies in this chain and common suppliers which are now moving production to the US from China

- Listing respective ports for the incoming supply by component and country/region.

- Calculating sea distances (in terms of nautical miles) between these ports and the average distance to and from the factory.

- Calculating the carbon footprint with the tons supplied and the distances travelled by these imports (from HS code-specific import export data and the FAF dataset)

- Vehicle cycle: raw materials + processing + manufacturing + assembly + operation + end of life

- Battery cycle: raw materials + transportation + processing + delivery + assembly (joins vehicle cycle at assembly point)

- Emissions = production + materials + transport + end of life

| # | Variable | Variable description | Equation | Unit | Calculated for | References |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | dij | Distance between locations i,j |  | km | mineral | Sea distances org |
| 2 | wij | Weight of mineral transported between locations i,j |  | kg | mineral | Cheng et.al, |
| 4 | W%ij | Weighted % of mineral transported | W%ij=wijWtotal | % | mineral |  |
| 5 | rij | Weight of mineral 1 kg battery |  | kg | battery | Greet |
| 6 |  |  | Contribution=W%ij×rij | kg | mineral | Greet |
| 7 | eα | Emission Factor for different transport modes α,β,γ | Eijα=dij .Contribution .(eα) | Kg CO2 eq | mineral | Ecoinvent Greet IPCC |
| 8 | Eij | Emission for mineral between location i,j by transport mode α,β,γ | Eij= Eijα+Eijβ+Eijγ | Kg CO2 eq | mineral |  |
| 9 | E | Total emissions for mineral transportation | E=Eij | Kg CO2 eq | mineral |  |
| 10 | T | Trade value | T= tij×contribution | $ / kg | mineral | WIT |
| 11 |  |  |  |  |  |  |
| HS Code: |  |  |  |  |  |  |
| 1 | Wij | Weight of commodity between location and US |  | kg | year | World Integrated Trade Solution |
| 2 | μ | Conversion factor (contained mineral per item) | wij=Wij . μ | kg | mineral | Cheng et.al, |
| 3 | dij | Distance between location and US |  | km | mineral | Sea distances org |
| 4 | w%ij | Weighted % of mineral transported | w%ij=wijwtotal | % | mineral |  |
| 5 | rij | Weight of mineral 1 kg battery |  | kg | battery | Greet |
| 6 | i | Import reliance |  | % | Mineral | US Mineral Commodity Summaries 2023 |
| 7 | Eij | Emission for mineral between location i,j by transport mode α,β,γ | Eij=rij . w%ij .i { dijα . eα+dijβ . eβ+dijγ . eγ } | Kg CO2 eq | mineral |  |
| 8 | Eus | Emissions in US domestically | Eus=rij . 1-i . dus . eβ | Kg CO2 eq | mineral |  |
| 9 | E | Total emissions for mineral transportation | E=Eij + Eus | Kg CO2 eq | mineral |  |